LCR and COMPANY // DTG Automation Breakdown Modules

Module A — Automated Area Calculation Engine

Definition

This module is the Area Extraction and Calculation System that automatically identifies and measures all drainage-relevant surface areas directly from Civil 3D 2025/2026 plan drawings, survey CSV data, and PDF plan sets.
It replaces the manual process of tracing impervious (paved, roofed) and pervious (grass, soil) regions and typing those areas into Excel drainage worksheets.

How it works

- Reads closed polylines and hatches from Civil 3D layers (e.g. parking, building, green area) using LCR’s .dwt template and naming conventions (IMP_SURFACE, CONC_DRIVE, TURF, etc.).

- Applies plan scale and coordinate reference automatically.

- Integrates survey CSV data (e.g. Storm Topo.csv, Storm Topo – IMPORT.csv) to confirm elevations, slopes, and drainage direction.

- Combines multiple building footprints into grouped basins or subcatchments as defined in grading plans.

- Exports structured outputs (CSV + GeoJSON) and populates the “TOC Calculation – Sunset Park.xlsx” workbook to update Time of Concentration (Tc) and Weighted C tables automatically.

- For PDF-based plans, uses OpenCV + OCR to visually extract scaled polygonal areas.

- Cross-links with Autodesk Storm & Sanitary Analysis (SSA) to supply area data directly into hydraulic simulations.

Example from LCR documents

In the L.J. Alleman Middle School (C-4 through C-8) grading sheets, engineers trace concrete, asphalt, and turf regions using Civil 3D’s polyline + hatch tools.
This module automatically computes total square footage, assigns C-values, and updates the TOC workbook — matching LCR’s current Excel process.

Value to LCR

- Reduces area takeoffs from hours to minutes.

- Eliminates manual scaling or transcription errors.

- Feeds Module C (DIA Engine) with live linked data.

- Standardizes templates and naming across AHS, Alleman, and future sites.

Module B — UDC & DOTD Specification Extraction Engine

Definition

This module is the Code and Specification Parsing System that reads municipal, parish, and DOTD drainage manuals (PDFs) and extracts structured data (tables, coefficients, rules) into a searchable database — forming LCR’s living specification library.

How it works

- Parses PDFs such as LCG Unified Development Code (UDC) and the DOTD Hydraulics Manual.

- Extracts:

  - Runoff coefficients (C-values) by land use

  - Rainfall intensity tables (I) from NOAA Atlas 14

  - Maximum Time of Concentration (Tc) limits

  - Detention pond release criteria

- Uses LangChain LLM parser to interpret headings like “Section 6.2.3 – Stormwater Design Standards.”

- Updates automatically when new code versions are uploaded.

- Stores data in PostgreSQL, queried by other modules via API.

- Includes optional parsing of project-specific specs (e.g. Architectural spec books from Deltek SpecPoint or subconsultant supplements).

Example from LCR documents

For Acadiana High DIA reports, rainfall intensities for 10-, 25-, 50-, 100-year storms and runoff coefficients come from LCG UDC and NOAA Atlas 14.
This module stores those values centrally so engineers never need manual lookup.

Value to LCR

- Centralizes all drainage standards.

- Ensures reports use the latest data (no outdated C-values).

- Cross-checks automatically in Module D (QA Checker).

- Supports custom specs for DOTD or project-specific addenda.

Module C — Drainage Impact Report Generator

Definition

The Drainage Impact Report Generator (DIA Engine) automatically creates complete drainage impact reports using live data from Modules A and B.

How it works

- Pulls area data (Module A) and UDC/DOTD parameters (Module B).

- Runs Rational Method (Q = CiA) for multiple storm frequencies (10, 25, 50, 100-year).

- Generates pre- and post-development discharges and comparison tables.

- Interfaces directly with Autodesk Storm & Sanitary Analysis (SSA) to import ASCII input files and export simulation results (hydrographs, tabular flows).

- Produces formatted Word and PDF reports styled after LCR’s Acadiana High templates (Exhibits 3A-3D).

- Allows re-running with updated plan data for instant revisions.

Example from LCR documents

Acadiana High DIA uses Exhibit 3A-3D tables showing Q results for each storm event.
This module recreates those automatically — no manual Excel work required.

Value to LCR

- Creates DIA reports in minutes.

- Ensures uniform layout and regulatory compliance.

- Removes manual SSA back-and-forth between Civil 3D and Excel.

- Supports rapid iteration when site plans change.

Module D — Plan Review & QA Automation

Definition

The Plan QA and Compliance Checker reviews every set of construction drawings (C-1 to C-18) to verify conformance with municipal and LCR standards before submittal. It functions as an “AI reviewer” for Civil 3D plans and PDFs.

How it works

- Reads all plan sheets and checks for required elements:
• Standard notes (LPDES, LUS, DOTD, ASTM, etc.)
• Sheet numbering and sequence (C-1 → C-18)
• Critical callouts, legends, title blocks
• Font, scale, and layout consistency

- Uses OCR and computer-vision recognition to cross-check text and detail references.

- Flags missing or incorrect items and generates a QA PDF with highlighted redlines.

- References content from standards like the AHS Early Release (C-11) sheet (ASTM grate standards) and L.J. Alleman notes, confirming presence and formatting.

Example from LCR documents

The AHS Early Release C-11 sheet includes ASTM references (“Grates/Solid Cover shall be Ductile Iron per ASTM A536 Grade 70-50-05”).
QA Automation detects missing notes before submission and records compliance status digitally.

Value to LCR

- Prevents city rejections for omissions or formatting errors.

- Standardizes plan set layout and checklists.

- Cuts internal QA time from 3–6 hours to minutes.

- Creates digital audit trail of QA checks.

Module E — Proposal & Document Automation

Definition

The Proposal and Document Generator automates creation of client proposals, submittal letters, and internal summaries based on pre-approved LCR templates.

How it works

- Pulls project metadata (client name, location, jurisdiction, project type).

- Retrieves technical inputs (C-values, rainfall data, etc.) from Modules A and B.

- Applies LCR branding and Dozier Tech Group layout for consistency.

- Generates ready-to-send Word / PDF files for:
• Fee proposals
• Drainage submittals
• Addenda and change orders
• Cover letters and summaries

- Optionally auto-populates signature fields and fee breakdown tables (hourly vs lump sum).

Example from LCR workflow

An engineer creating a new drainage study (e.g. “L.J. Alleman Additions”) enters the project metadata once; the system fetches the correct UDC references, applies the template, and produces a professional cover letter and fee summary automatically.

Value to LCR

- Speeds agency correspondence and submittals.

- Maintains consistent tone and format across projects.

- Reduces time spent on Word formatting.

- Improves presentation and professional branding.

Overall Workflow Summary

- Start with raw project data (Civil 3D DWGs + survey CSV + Excel TOC).

- Module A extracts areas and updates Excel inputs.

- Module B retrieves and stores UDC/DOTD specifications and manual data.

- Module C runs Rational Method calculations and generates DIA reports (Word/PDF).

- Module D performs automated QA and compliance checks on plan sets.

- Module E creates cover letters, fee proposals, and submittals for delivery.

Each module feeds the next, creating a seamless pipeline from raw design files → verified city-ready reports and submittals.